Metadata design for integrating CEOP satellite imagery, reference site data and simulation result data

Version 1.0

Shibasaki Group

For “Koike global water informatics” project

October 30, 2004

# Center for Spatial Information Science
The University of Tokyo
4-6-1 Komaba, Meguro-ku, Tokyo 153-8505, Japan
Tel: +81-3-5452-6412
Fax: +81-3-5452-6414

# http://shiba.iis.u-tokyo.ac.jp

To integrate various satellite data, in-situ observation data and model simulation results like MOLTS, it is necessary to exactly represent or reconstruct geometric conditions in observing or acquiring data by space-borne sensors. Information to describe or reconstruct the observation conditions is usually represented by metadata. Standardization on metadata of satellite imagery is being conducted by several organizations such as ISO/TC211. Those metadata, however, do not meet all the requirements in integrating satellite imagery, in-situ observation data and 4D model simulation results collected in CEOP. This report reviews the present status of documents for standardization by ISO and associated organizations and proposes complementary components to fulfill the requirements for integrating data by CEOP.

For more information or questions on this report, please feel free to contact:

# Shibasaki Group

# Center for Spatial Information Science
The University of Tokyo
4-6-1 Komaba, Meguro-ku, Tokyo 153-8505, Japan

Telephone: +81-3-5452-6412
Facsimile:  +81-3-5452-6414

# Email: {shiba, xierong}@skl.iis.u-tokyo.ac.jp

WWW Home Page: http://shiba.iis.u-tokyo.ac.jp

Table of contents

1. Introduction	 (4)

2. Metadata development	 (5)

2.1 ISO 19115 core metadata for geographic datasets	 (5)

2.2 ISO 19139 metadata implementation	 (5)

2.3 CEOP/ISO metadata standard harmonization	 (7)

2.4 Metadata output format	 (8)

3. Satellite imagery matadata	 (9)

3.1 AMSR-E satellite metadata as an example	 (9)

3.1.1 AMSR-E satellite imagery data set	 (9)

3.1.2 Header information of data file and their mapping to ISO metadata	 (11)

3.1.3 Example of satellite metadata	 (11)

4. Reference site metadata	 (17)

4.1 Reference site data set	 (17)

4.1.1 Surface meteorological and radiation data	 (17)

4.1.2 Flux data	 (19)

4.1.3 Soil temperature and soil moisture data	 (19)

4.1.4 Meteorological tower data	 (20)

4.2 Mapping CEOP reference site metadata and ISO 19115 metadata	 (21)

5. GRIB metadata	 (26)

5.1 WMO core metadata	 (26)

5.2 Example of GRIB metadata	 (30)

6. MOLTS metadata	 (33)

6.1 GMAO MOLTS metadata	 (33)

6.2 MOLTS metadata	 (34)

References	 (40)

FIGURES

# Figure 1. Scope of the report	 (4)

Figure 2. Metadata package	 (6)

Figure 3. Mapping CEOP metadata to ISO metadata	 (8)

Figure 4. Data structure of AMSR-E level 1 product	 (10)

Figure 5. Data structure of AMSR-E level 2 product	 (10)

Figure 6. Data structure of AMSR-E level 3 product	 (11)

TABLES

Table 1. Core metadata for geographic datasets	 (7)

Table 2. Types of AMSR-E products	 (9)

Table 3. Header information of data file for CEOP satellite data	 (13)

## Table 4. Reference site data set	 (17)

## Table 5. Format description of surface meteorological and radiation data	 (18)

Table 6. Format description of flux data	 (19)

Table 7. Format description of soil temperature and soil moisture data	 (19)

Table 8. Format description of meteorological tower data	 (20)

# Table 9. Mapping CEOP reference site metadata and ISO 19115 metadata	 (22)

Table 10. Additional items for CEOP reference site metadata	 (25)

Table 11. WMO core metadata	 (26)

Table 12. MOLTS variables to ISO mapping	 (36)

# 1. Introduction

# To integrate various satellite data, reference site observation data and multi-dimensional model simulation results like GRIB and MOLTS, it is necessary to exactly represent or reconstruct geometric conditions in observing or acquiring data by space-borne sensors. Information to describe or reconstruct the observation conditions is usually represented by metadata. Standardization on metadata of satellite imagery is being conducted by several organizations such as ISO/TC211 (http://www.isotc211.org/). This report reviews the present status of documents for standardization by ISO and proposes complementary components to fulfill the requirements for integrating satellite imagery, reference site observation data and model simulation results collected in the Coordinated Enhanced Observing Period (CEOP) (http://ceop.net/).

For the design of metadata, two stages can be identified as follows;

(1) Finding data

(2) Integrating data

In the “finding data” stage, users identify data which may be useful for their own purposes. Metadata are required to provide minimum information to support the users’ judgment or evaluation of usability of the data. In the “integrating data” stage, users overlay spatially and temporally the data so that the spatio-temporal relationships among data and/or pixels can be explicitly reconstructed. For this purpose, metadata should include enough information to let users overlay the retrieved data. In case of satellite data, if they are not geometrically and radiometrically corrected, metadata should provide enough information for geometric correction and radiometric correction such as satellite orbit, sensor attitude and many calibration parameters.

In case of CEOP reference site data, because all satellite data are already geocoded and radiometrically corrected, i.e. satellite image data are already converted to raster data, no further data for geometric and radiometric correction need be included in the metadata. This means that the same metadata can be used for “data-finding” and “data integration” as long as CEOP reference site data are concerned. This report summarizes metadata design for CEOP reference site data. But this metadata can be applied to the data-finding stage of the other satellite data, reference site observation data and model simulation results.

# (The authors are providing the second report on the metadata design focusing on “data integration” stage of satellite images based on ISO 19130 document.)

# Figure 1. Scope of the report

# 2. Metadata development

Metadata, commonly defined as "data about data", is a structured set of information which describes data (including both digital and non-digital datasets) stored in administrative systems. Metadata may provide a short summary about content, purpose, quality, location of data as well as information related to its creation.

Standardized metadata support users in effectively and efficiently accessing data by using a common set of terminology and metadata elements that allow for a quick means of data discovery and retrieval through metadata clearinghouses. ISO 19115 metadata standard [1] Geographic Information  Metadata was approved by the international community in April 2003 as a tool to define metadata in the field of geographic information. This standard defines the schema required for describing geographic information and services and provides information about identification, extent, quality, spatial and temporal schema, spatial reference, and distribution of digital geographic data. It specifies the comprehensive standard containing 14 packages and about 400 metadata elements. Its schema is described in UML and detail specifications are defined in the data dictionary. Users may describe not only the comprehensive metadata but also core metadata for the discovery of geographic data. Currently, ISO TC211 is now discussing ISO 19139: Geographic information – Metadata – Implementation specifications [2] for the implementation of metadata.

## 2.1 ISO 19115 core metadata for geographic datasets

In ISO 19115 standard, metadata for geographic data is presented in UML packages, including: metadata entity set information, identification information, constraint information, data quality information, maintenance information, spatial representation information, reference system information, content information, portrayal catalogue information, distribution information, metadata extension information, application schema information, units of measurement, extent information, citation and responsible party information.  Figure 2 illustrates the layout of the packages. Each package contains one or more entities (UML classes), which can be specified (subclassed) or generalized (superclassed). Entities contain elements (UML class attributes) which identify the discrete units of metadata.  Entities may be related to one or more other entities.  They can be aggregated and repeated as necessary to meet: (1) the mandatory requirements stated in the standard; and (2) additional user requirements.

In the standard, an extensive set of metadata elements is defined; typically only a subset of the full number of elements is used. However, it is essential that a basic minimum number of metadata elements be maintained for a dataset. Table 1 lists the core metadata elements required to identify a dataset, typically for catalogue purposes. This list contains metadata elements answering the following questions: “Does a dataset on a specific topic exist (‘what’)?”, “For a specific place (‘where’)?”, “For a specific date or period (‘when’)?” and “A point of contact to learn more about or order the dataset (‘who’)?”. Using the recommended optional elements in addition to the mandatory elements will increase interoperability, allowing users to understand without ambiguity the geographic data and the related metadata provided by either the producer or the distributor. Dataset metadata profiles of the standard shall include this core. Listed in the table are the core metadata elements (mandatory and recommended optional) required for describing a dataset. A “M” indicates that the element is mandatory. An “O” indicates that the element is optional. A “C” indicates that the element is mandatory under certain conditions.

## 2.2 ISO 19139 metadata implementation

ISO/TC 211 has approved a project to develop ISO 19139, Geographic information - Metadata - Implementation specification. The ISO 19139 specification defines an implementation UML model based upon the abstract UML model in ISO 19115. The UML implementation model may be used in conjunction with an XML schema, which will be included in an informative annex to ISO 19139, to describe digital geographic datasets.

The XML schema describes structure and content of the mandatory and optional elements in the UML metadata model. This schema can be used to validate an XML instance document conformance to the UML metadata implementation model. Transforming from UML profile to XML schema includes two steps as follows,

(1)  Output XML to be used to generate XSD

Once the UML model is representative of a geographic metadata dataset implementation, it is necessary to derive the XML schemas directly from the model. The first step in this phase is to run XMLout4XMLSchema.ebs to create an XML document that is used to eventually create an XML schema. XMLout4XMLSchema.ebs is not specific to ISO 19115 and it can be used to generate various XML schema based on the selected package in Rational Rose. To create the schema represented in this specification, XMLout4XMLSchema.ebs is executed for each custom schema presented. For the sake of describing the creation process, the output xml files from those respective steps are referred to as iso19115_template.xml and theConstraintTypes_template.xml, although the actual filenames are irrelevant to the process. These XML files are interim templates used to generate the final XML schemas (XSD's) that are presented in this specification.

(2) Transforming XML to XSD

To generate the final XML schemas, it is necessary to perform an XSL transformation on the aforementioned xml template files. An XSL transformation is simply an XML application that specifies the rules by with one XML document is transformed into another. Using this process the templates output from the UML models are converted to XML schemas which are another type of XML document. The transformation rules applicable to this step are specified in UMLSchema_To_XMLSchema_xs.xslt.

Figure 2. Metadata package

Table 1. Core metadata for geographic datasets

| Dataset title (M) (MD_Metadata > MD_Identification.citation > CI_Citation.title) | Spatial representation type (O) (MD_Metadata > MD_DataIdentification.spatialRepresentationType) |
| --- | --- |
| Dataset reference date (M) (MD_Metadata > MD_DataIdentification.citation > CI_Citation.date) | Reference system (O) (MD_Metadata > MD_ReferenceSystem) |
| Dataset responsible party (O) (MD_Metadata > MD_Identification.pointOfContact > CI_ResponsibleParty) | Lineage statement (O) (MD_Metadata > DQ_DataQuality > LI_Lineage.statement) |
| Geographic location of the dataset (by four coordinates or by geographic identifier) (C) (MD_Metadata > MD_DataIdentification.extent > EX_GeographicBoundingBox or MD_DataIdentification.extent > EX_GeographicDescription ) | On-line resource (O) (MD_Metadata > MD_Distribution > MD_DigitalTransferOption.onLine > CI_OnlineResource) |
| Dataset language (M) (MD_Metadata > MD_DataIdentification.lauguage) | Metadata file identifier (O) (MD_Metadata.fileIdentifier) |
| Dataset character set (C) (MD_Metadata > MD_DataIdentification.characterSet) | Metadata standard name (O) (MD_Metadata.metadataStandardName) |
| Dataset topic category (M) (MD_Metadata > MD_DataIdentification.topicCategory) | Metadata standard version (O) (MD_Metadata.metadataStandardVersion) |
| Spatial resolution of the dataset (O) (MD_Metadata > MD_DataIdentification.spatialResolution > MD_Resolution.equivalentScale or MD_Resolution.distance) | Metadata language (C) (MD_Metadata.language) |
| Abstract describing the dataset (M) (MD_Metadata > MD_Identification.abstract) | Metadata character set (C) (MD_Metadata.characterSet) |
| Distribution format (O) (MD_Metadata > MD_Distribution > MD_Format.name and MD_Format.version) | Metadata point of contact (M) (MD_Metadata.contact > CI_ResponsibleParty) |
| Additional extent information for the dataset (vertical and temporal) (O) (MD_Metadata > MD_DataIdentification.extent > verticalElement or MD_Metadata > MD_DataIdentification.extent > temporalElement) | Metadata date stamp (M) (MD_Metadata.dateStamp) |

## 2.3 CEOP/ISO metadata standard harmonization

As shown in Figure 3, the "mapping" is required to define between CEOP metadata and ISO metadata. By this mapping, some items can be reflected in ISO core metadata, and some are reflected in ISO non-core metadata. To fill the gap between the existing CEOP metadata and the ISO metadata, some metadata are added in the CEOP. These additional items will be helpful for use to decide the availability of data sets.

Figure 3. Mapping CEOP metadata to ISO metadata

## 2.4 Metadata output format

Data interchange is an essential element in achieving interoperability between heterogeneous systems. Metadata may be created, stored, and used in a variety of formats. There are usually two kinds of fundamental interoperability format.

(1) UML model  semantic interoperability, is to define and agree on the semantics of the content and logical structures of data. UML model is an intuitive and powerful choice for describing the semantics and logical structures of data as well as behavioral aspects.

(2) XML schema  syntactic interoperability, is to define a system and platform independent data structure that can represent data corresponding to the application schema. XML can be chosen as a data interchange format because of its simplicity, Web conformance, and extensively tool support.

# 3. Satellite imagery metadata

## 3.1 AMSR-E satellite metadata as an example

AMSR-E is chosen as an illustrative example for the metadata structure.

### 3.1.1 AMSR-E satellite imagery data set

AMSR-E is the radio sensors that precisely measure faint radio waves radiated from the earth surface and atmosphere by themselves with multiple frequency bands. AMSR-E product level is defined as shown in the Table 2 [3].

Table 2. Types of AMSR-E products

| Product name | Outline of the product |
| --- | --- |
| Level 1A | The product contains the radiometrically and geometrically processed data, such as the observed count data, the antenna temperature coefficient, the geolocations of observed points, which are calculated from observed raw AMSR-E data. |
| Level 1B | The brightness temperature of each observed point are calculated from L1A data and stored into L1B product together with other ancillary data. (Not applicable for NSIDC DAP package) |
| Level 1B Map | Map projected products calculated from L1B product. The mapping methods are latitude/longitude equatorial, mercator, or polar stereo. (Not applicable for NSIDC DAP package) |
| Level 2 | The water related physical parameters (accumulated water vapor, accumulated cloud liquid water, amount of precipitation, sea surface wind speed, sea surface temperature, sea ice concentration, snow water equivalence, and soil moisture) are calculated from L1B product to produce L2 products. Separate L2 products are produced for each physical parameter. |
| Level 2 Map | Map projected products extracted from L2 products. |
| Level 3 | The water related parameters are averaged daily or monthly upon the pre-defined grids on the Earth. |

(1) AMSR-E level 1 product

Level 1A product stores the observed antenna temperature count data of the corresponding position (longitude, latitude of the observation points), the navigation data, the attitude data and the other useful data for the analysis of AMSR-E data. Level 1B product stores the observed brightness temperature data of the corresponding position (longitude, latitude of the observation points), the navigation data, the attitude data and the other useful data for the analysis of AMSR-E data. Level 1B Map product stores the observed brightness temperature data is projected by user specified mapping and resampling method.

Level 1 product consists of the header part and data part. The header part includes the core metadata and the product metadata. The core metadata shows the outline of its product and the product metadata shows the main characteristics of AMSR-E and the telemetry conversion table. In the data part, the observed antenna count data, its position data and so on, are stored with SDS, Vdata format. The structure of AMSR-E Level 1 product is shown in Figure 4. As shown in the figure, the core metadata are stored as a header with a global attribute model. The detailed items stored in the core metadata and the product metadata are shown in Tables 3.

(2) AMSR-E level 2 product

Level 2 product is processed from level 1B product. It contains geophysical quantities, such as water vapor, cloud liquid water, precipitation, sea surface wind speed, sea surface temperature, sea ice concentration, snow water equivalent, and soil moisture, which are calculated from the brightness temperature. Level 2 Map product has projection of level 2 data on a map. Its data is selected from level 2 data by specified parameters. Number of image in the level 2 Map product is 300 pixels × 300 pixels. Each pixel is 2 bytes. This data is stored as SDS in the product.

(a) Level 1A. 			(b) Level 1B.			 (c) Level 1B Map.

Figure 4. Data structure of AMSR-E level 1 product

Level 2 product contains two major parts: the header and data. The header part is composed of core metadata. Core metadata describes the information about a product. The geophysical quantity in level 2 product is stored in SDS, 196 ×1975 (nominal), 2 dimension array. (Level 1B product has two overlaps with before or next scan. On the other hand,  the overlaps are deleted in level 2 product for a half orbit. The structure of AMSR-E Level 2 product is shown in Figure 5. As shown in the figure, the core metadata are stored as a header with a global attribute model. The detailed items stored in the core metadata and the product metadata are shown in Tables 3.

(a) Level 2.		                 (b) Level 2Map.

Figure 5. Data structure of AMSR-E level 2 product

(3) AMSR-E level 3 product

Level 3 product is processed from level 1B or level 2 product. Level 3 product contains the global data which are the daily or monthly average data of each geophysical quantity, such as brightness temperature in level 1B product, WV, CLW, AP, SSW, IC, SWE, or SM in level 2 product. The data in level 3 is calculated average daily or monthly in each directions, such as ascending or descending, and projected by equirectangular or polar stereo graphic. In the case of polar stereo the data more than 60 degrees for latitude are projected.

There are 3 types for storing the average in level 3 product. In the case of equirectangular, the data is stored as a SDS of 2 dimension array, which is 304 points for pixel and 448 for line. In Polar stereo, it is 316 for pixel and 448 for line, or 431 for pixel and 573 for line. In the case of polar stereo of TB or IC, the only average around the north pole or south is stored in SDS, for SWE the only the average around the north pole is stored. As above the data part is composed of only one SDS.

The structure of AMSR-E Level 3 product is shown in Figure 6. As shown in the figure, the core metadata are stored as a header with a global attribute model. The detailed items stored in the core metadata and the product metadata are shown in Tables 3.

Figure 6. Data structure of AMSR-E level 3 product

### 3.1.2 Header information of data file and their mapping to ISO metadata

Header information of data file contains the necessary information about the product. These items are selected from the necessary attributes. The metadata is stored in the core metadata and its name is considered as global attribute.  Metadata in each global attribute is preserved in ASCII. A list of core metadata designed for AMSR-E satellite data is shown in Table3.

### 3.1.3 Example of satellite metadata [9]

MD_Identification:

citation:

title: F13SSMI1B_200107152131_wpo_low

MD_Sensor:

title: F13 SSM/I

MD_DataIdentification:

topicCategory

MD_Keywords:

Brightness Temperature

EX_TemporalExtent:

extent: 2001/07/15 21:31:18

MD_GridSpatialRepresentation:

numberOfDimensions

axisDimensionProperties

cellGeometry

MD_RangeDimension:

sequenceIdentifier

MD_Band:

CellValueType: 2byte integer

MD_CoverageDescription:

attributeDescription: Kelvin

MD_Band:

scaleFactor: 10

MD_RangeDimension:

descriptor: 19.35GHz-V,19.35GHz-H,22.235GHz-V,37.0GHz-V,37.0GHz-H

MD_Band:

bandResolution

CI_Citation:

title: WesternPacificOcean

MD_DataIdentification:

geographicDescription

MD_DataIdentification:

EX_GeographicBoundingBox:

WestBoundLongitude

EastBoundLongitude

SouthBoundLatitude

northBoundLatitude

SD_PointPosition:

Origin

MD_GridSpatialRepresentation

gridSize: 0.25

MD_Band:

valueForMissingData: 9999

outOfObservation

MD_ImageDescription:

obseavationAreaRatio: 100

LI_ProcessStep:

description: Software Ver:1.0

description

LI_ProcessStep:

dateTime: 2003/07/03

LI_ProcessStep:

processor: NASDA/EORC@HARUMI

LI_Source:

description: f13_Tb_01196_25D.hdf

CI_ResponsibleParty

originalProcessingCenter: GHRC@GHCC

MD_Format:

Name

Version

Blank

Table 3. Header information of data file for CEOP satellite data

| Index | CEOP header data item description | ISO metadata package | Class | Attribute | Data type | ### Explanations |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Filename | Identification information | MD_Identification | citation | CI_Citation | Filename can be title (an attribute of CI_Citation class). See 19115_No.360 |
| 2 | Sensor |  | MD_Sensor |  |  | If MD_Sensor class suggested in 19115-2 is not applied, this will be handled as "title". |
| 3 | Product |  | MD_DataIdentification | topicCategory | Class | High-level geographic data thematic classification to assist in the grouping and search of available geographic data sets. Can be used to group keywords as well. Listed examples are not exhaustive. "imageryBaseMapsEarthCover"seems to be the most appropriated among the examples given. But, "CEOP_Satellite_Imagery" is suggested as a better term. |
|  |  |  | or MD_Keywords | keywords | CharacterString | Keywords can be used. |
| 4 | Observation Date and Time | Extent information | EX_TemporalExtent | Class | TM_Primitive | Date and time for the content of the dataset |
| 5 | Image Size | Spatial representation information | MD_GridSpatialRepresentation |  |  | Spatio grid dimension |
|  |  |  |  | numberOfDimensions | Integer |  |
|  |  |  |  | axisDimensionProperties | Sequence<MD_Dimension> | Row, column, vertical etc. see B.5.14 MD_DimensionNameTypeCode <<CodeList>> Resolution of Grid is also represented here |
|  |  |  |  | cellGeometry | class (MD_CellGeometryCode) | Point or area |
|  |  | Content information | MD_RangeDimension | sequenceIdentifier | MemberName | Number of spectral/frequency bands |
| 6 | Data Type | Content information | MD_Band | cellValueType |  | e.g. ２byte Integer Proposed in 19115-2: mandatory (if applicable) bit representation of data value in raster cell |
| 7 | Data Unit | Content information | MD_CoverageDescription | attributeDescription | record type (see. 19103) | Unit of physical measurement Def. Record type (see. 19103): "A Record is used as an implementation representation for features, by keeping a list of (name, value) pairs in a dictionary. This represents a generic storage structure for features." |
| 8 | Scale Factor | Content information | MD_Band | scaleFactor | Real | 19115_No.266 |
| 9 | Observation Channel | Content information | MD_RangeDimension | descriptor | CharacterString |  |
| new | FOV (new) |  | MD_Band | bandResolution |  | Proposed by 19115-2: smallest distance between which separate points can be distinguished, as specified in instrument design. |
| 10 | Reference Site | Citation and responsible party | CI_Citation | title | CharacterString | 19115_No.360 (to be included in "title" of file). |
|  |  | Identification | MD_DataIdentification | geographicDescription | EX_geographicDescription --> MD_Identifier | Reference sites to be registered as a namespace. |
| 11 | LAT./LON. In center of Lower Left Pixel | Identification --> Spatial representaion information | MD_DataIdentification --> EX_GeographicBoundingBox | westBoundLongitude, eastBoundLongitude, southBoundLatitude,. northBoundLatitude |  | 19115_No.343. |
| new | LAT./LON. And Heght of the Satellite in scanninng the Scene Center (new) | MD_Sensor? | SD_PointPosition | origin | GM_Point | Definition: position of satellite when scanned the ceter of a scene (Lat. Lon. Height (km)) 19130 "sensor model" allows to describe the position of a satellite at any arbitrary time with SD_PointPosition. But it is not included by 19115-2. Need to extend 19115-2 according to 19130. |
| 12 (modified) | Grid Size | Spatial representation information | MD_GridSpatialRepresentation |  |  |  |
| 13 | Missing Value | Content information | MD_Band | new attribute should be added "value_for_missing_data" | CharacterString | Missing data: data are not captured though a grid-cell is in the observation area. |
|  |  | Content information | MD_Band | new attribute should be added "out_of_observation" | boolean | Out of observation: a grid-cell is out of the observation area. |
| 14 | Observation area ratio (Ovserved Pixel/All Pixel) | Content information | MD_ImageDescription | new attribute should be added. | Real | Need to Add. |
| 15 | Subset software version | Data quality information (Lineage information) | LI_ProcessStep | description | CharacterString |  |
| new | Reference for Processing Algorithm for Physical data product | Data quality information (Lineage information) | LI_ProcessStep | description | CharacterString | Citation information of documents describing algorithms of computing physical parameter values from satellite imagery to be described here. |
| 16 | Processing Date | Data quality information (Lineage information) | LI_ProcessStep | dateTime | DateTime |  |
| 17 | Processing Center | or Data quality information (Lineage information) | LI_ProcessStep | processor | class: CI_ResponsibleParty |  |
| 18 | Input Original Filename | Data quality information (Lineage information) | LI_Source | description | CharacterString |  |
|  | Original File version | Data quality information (Lineage information) | LI_Source | description | CharacterString |  |
| 19 | Original File Processing Center | Data quality information (Lineage information) | CI_ResponsibleParty and/or CI_Contact |  | CharacterString |  |
| 20 | HDF library versionno longer relevant | Distribution information | MD_Format | name and version | CharacterString | This data item will be removed. |
| 21 | Blank |  |  |  |  |  |

# 4. Reference site metadata

## 4.1 Reference site data set

Table 4 shows the CEOP reference site data, which includes four resolution data sets: (1) surface meteorological and radiation data set; (2) flux data set; (3) soil temperature and soil moisture data set; (4) meteorological tower data set.

Table 4. Reference site data set

| Data set | Data category | Submission date (First half EOP-3) | Submission date (Second half EOP-3) | Submission date (First half EOP-4) | Submission date (Second half EOP-4) | Detailed format description |
| --- | --- | --- | --- | --- | --- | --- |
| Surface meteorological and radiation data set | Category 1 | 1 October 2003 | 1 April 2004 | 1 October 2004 | 1 April 2005 | Surface meteorological and radiation format |
| Flux data set | Category 2 | 1 July 2004 | 1 January 2005 | 1 July 2005 | 1 January 2006 | Flux format |
| Soil temperature and soil moisture data set | Category 1 | 1 October 2003 | 1 April 2004 | 1 October 2004 | 1 April 2005 | Soil temperature and soil moisture format |
| Meteorological tower data set | Category 1 | 1 October 2003 | 1 April 2004 | 1 October 2004 | 1 April 2005 | Meteorological tower format |

### 4.1.1 Surface meteorological and radiation data [10]

This CEOP dataset contains the surface meteorological and radiation parameters reported by each CSE/Reference Site/Station Identifier combination at each Date/Time. The surface meteorological and radiation dataset format contains 8 metadata parameters, 38 data parameters and flags. The metadata parameters describe the station location and time at which the data were collected. The flag values are assigned by the reference site, according to the CEOP specified data flags. The following is a sample record of the surface meteorological and radiation data. Table 5 details the space separated data parameters and associated flags in each record.

2001/07/01  01:00  2001/07/01  01:00  LBA  Pantanal  Pantanal  -19.56339  -57.01494  -999.99  1003.30  U  25.62  U 16.54  U  57.22  U  11.75  U  2.98  U  38.82  U  -1.87  U  -2.32  U  0.00  U  -999.99  M  -2.40  U  -1.26  U  -999.99  M  -999.99  M  -27.10  U  -999.99  M  -0.52  U  -3.22  U

## Table 5. Format description of surface meteorological and radiation data

| Parameter | C format | Missing value | Final units/equations/notes |
| --- | --- | --- | --- |
| UTC Nominal Date/Time | 16 chars | N/A | yyyy/mm/dd HH:MM, where MM is 00 or 30, only |
| UTC Actual Date/Time | 16 chars | N/A | yyyy/mm/dd HH:MM |
| CSE Identifier | 10 chars | N/A | Fill name with underscores, not spaces. |
| Reference Site Identifier | 15 chars | N/A | Fill name with underscores, not spaces. |
| Station Identifier | 15 chars | N/A | Fill name with underscores, not spaces. |
| Latitude | f10.5 | -99.99999 | decimal degrees. South is negative. |
| Longitude | f11.5 | -999.99999 | decimal degrees. West is negative. |
| Elevation | f7.2 | -999.99 | meters |
| Station Pressure | f7.2 | -999.99 | hPa (mb). |
| Station Pressure Flag | 1 char | M | See Flag values. |
| Air Temperature | f7.2 | -999.99 | Celsius. |
| Air Temperature Flag | 1 char | M | See Flag values. |
| Dew Point Temperature | f7.2 | -999.99 | Celsius. See Equations |
| Dew Point Temperature Flag | 1 char | M | See Flag values. |
| Relative Humidity | f7.2 | -999.99 | percent. See Equations |
| Relative Humidity Flag | 1 char | M | See Flag values. |
| Specific Humidity | f7.2 | -999.99 | g/kg. See Equations |
| Specific Humidity Flag | 1 char | M | See Flag values. |
| Wind Speed | f7.2 | -999.99 | m/s. See Equations |
| Wind Speed Flag | 1 char | M | See Flag values. |
| Wind Direction | f7.2 | -999.99 | degrees. See Equations |
| Wind Direction Flag | 1 char | M | See Flag values. |
| U Wind Component | f7.2 | -999.99 | m/s. See Equations |
| U Wind Component Flag | 1 char | M | See Flag values. |
| V Wind Component | f7.2 | -999.99 | m/s. See Equations |
| V Wind Component Flag | 1 char | M | See Flag values. |
| Precipitation | f7.2 | -999.99 | millimeters. Incremental precipitation over the previous 30 minutes. |
| Precipitation Flag | 1 char | M | See Flag values. |
| Snow Depth | f7.2 | -999.99 | centimeters. |
| Snow Depth Flag | 1 char | M | See Flag values. |
| Incoming Shortwave | f8.2 | -999.99 | W/m2. |
| Incoming Shortwave Flag | 1 char | M | See Flag values. |
| Outgoing Shortwave | f8.2 | -999.99 | W/m2 |
| Outgoing Shortwave Flag | 1 char | M | See Flag values. |
| Incoming Longwave | f8.2 | -999.99 | W/m2. |
| Incoming Longwave Flag | 1 char | M | See Flag values. |
| Outgoing Longwave | f8.2 | -999.99 | W/m2. |
| Outgoing Longwave Flag | 1 char | M | See Flag values. |
| Net Radiation | f8.2 | -999.99 | W/m2. See Equations |
| Net Radiation Flag | 1 char | M | See Flag values. |
| Skin Temperature | f8.2 | -999.99 | Celsius. |
| Skin Temperature Flag | 1 char | M | See Flag values. |
| Incoming PAR | f8.2 | -999.99 | µmol/m2/s. |
| Incoming PAR Flag | 1 char | M | See Flag values. |
| Outgoing PAR | f8.2 | -999.99 | µmol/m2/s. |
| Outgoing PAR Flag | 1 char | M | See Flag values. |

### 4.1.2 Flux data [11]

This CEOP dataset contains the flux parameters reported by each CSE/Reference Site/Station Identifier combination for each depth at each Date/Time. The flux dataset format contains 9 metadata parameters, 8 data parameters and flags. The metadata parameters describe the station location and time at which the data were collected. The flag values are assigned by the reference site, according to the CEOP specified data flags. The following is a sample record of the flux data. Table 6 details the space separated data parameters and associated flags in each record.

2001/07/01  01:00  2001/07/01  01:00  LBA  Pantanal  Pantanal  -19.56339  -57.01494  -999.99  0.00  -83.42  U  -999.99  M  -999.99  M  -999.99  M  2001/07/01  01:00  2001/07/01  01:00  LBA  Pantanal  Pantanal  -19.56339  -57.01494  -999.99  -0.02  -999.99  M  -999.99  M  -999.99  M  -9.59  U

## Table 6. Format description of flux data

| Parameter | C format | Missing value | Final units/equations/notes |
| --- | --- | --- | --- |
| UTC Nominal Date/Time | 16 chars | N/A | yyyy/mm/dd HH:MM, where MM is 00 or 30, only |
| UTC Actual Date/Time | 16 chars | N/A | yyyy/mm/dd HH:MM |
| CSE Identifier | 10 chars | N/A | Fill name with underscores, not spaces. |
| Reference Site Identifier | 15 chars | N/A | Fill name with underscores, not spaces. |
| Station Identifier | 15 chars | N/A | Fill name with underscores, not spaces. |
| Latitude | f10.5 | -99.99999 | decimal degrees. South is negative. |
| Longitude | f11.5 | -999.99999 | decimal degrees. West is negative. |
| Elevation | f7.2 | -999.99 | meters |
| Sensor Height | f7.2 | -999.99 | meters; Height of sensor. Positive above ground level. Negative below ground. |
| Sensible Heat Flux | f8.2 | -999.99 | W/m2 |
| Sensible Heat Flux Flag | 1 char | M | See Flag values. |
| Latent Heat Flux | f8.2 | -999.99 | W/m2 |
| Latent Heat Flux Flag | 1 char | M | See Flag values. |
| CO2 Flux | f8.2 | -999.99 | µmol/m2/s |
| CO2 Flux Flag | 1 char | M | See Flag values. |
| Soil Heat Flux | f8.2 | -999.99 | W/m2; |
| Soil Heat Flux Flag | 1 char | M | See Flag values. |

### 4.1.3 Soil temperature and soil moisture data [12]

This CEOP dataset contains the soil temperature and soil moisture parameters reported by each CSE/Reference Site/Station Identifier combination for each depth at each Date/Time. The CEOP soil temperature and soil moisture dataset format contains 9 metadata parameters, 4 data parameters and flags. The metadata parameters describe the station location and time at which the data were collected. The flag values are assigned by the reference site, according to the CEOP specified data flags. The following are sample records of the soil temperature and soil moisture data. Table 7 details the space separated data parameters and associated flags in each record.

2001/07/01  00:00  2001/07/01  00:00  CAMP  Mongolia  107  45.73983  106.65153  1342.00  -0.03  17.76  U  5.20  U 2001/07/01  00:00  2001/07/01  00:00  CAMP  Mongolia  107  45.73983  106.65153  1342.00  -0.10  16.30  U  6.46  U

## Table 7. Format description of soil temperature and soil moisture data

| Parameter | C format | Missing value | Final units/equations/notes |
| --- | --- | --- | --- |
| UTC Nominal Date/Time | 16 chars | N/A | yyyy/mm/dd HH:MM, where MM is 00 or 30, only |
| UTC Actual Date/Time | 16 chars | N/A | yyyy/mm/dd HH:MM |
| CSE Identifier | 10 chars | N/A | Fill name with underscores, not spaces. |
| Reference Site Identifier | 15 chars | N/A | Fill name with underscores, not spaces. |
| Station Identifier | 15 chars | N/A | Fill name with underscores, not spaces. |
| Latitude | f10.5 | -99.99999 | decimal degrees. South is negative. |
| Longitude | f11.5 | -999.99999 | decimal degrees. West is negative. |
| Elevation | f7.2 | -999.99 | meters |
| Sensor Height | f7.2 | -999.99 | meters; Height of sensor. Positive above ground level. Negative below ground. |
| Soil Temperature | f8.2 | -999.99 | Celsius |
| Soil Temperature Flag | 1 char | M | See Flag values. |
| Soil Moisture | f8.2 | -999.99 | percent. Volumetric water content. |
| Soil Moisture Flag | 1 char | M | See Flag values. |

### 4.1.4 Meteorological tower data [13]

This CEOP meteorological tower dataset contains the meteorological tower parameters reported by each CSE/Reference Site/Station Identifier combination for each height at each date/time. The CEOP meteorological tower dataset format contains 9 metadata parameters, 18 data parameters and flags. The metadata parameters describe the station location and time at which the data were collected. The flag values are assigned by the Reference Site, according to the CEOP-specified data flags. The following is a sample record of the meteorological tower data. The Table 8 details the space separated data parameters and associated flags in each record.

2001/07/01  01:00  2001/07/01  01:00  LBA  Pantanal  Pantanal  -19.56339  -57.01494  -999.99  0.00  1003.30  U  25.62  U  16.54  U  57.22  U  11.75  U  2.98  U  38.82  U  -1.87  U  -2.32  U

## Table 8. Format description of meteorological tower data

| Parameter | C format | Missing value | Final units/equations/notes |
| --- | --- | --- | --- |
| UTC Nominal Date/Time | 16 chars | N/A | yyyy/mm/dd HH:MM, where MM is 00 or 30, only |
| UTC Actual Date/Time | 16 chars | N/A | yyyy/mm/dd HH:MM |
| CSE Identifier | 10 chars | N/A | Fill name with underscores, not spaces. |
| Reference Site Identifier | 15 chars | N/A | Fill name with underscores, not spaces. |
| Station Identifier | 15 chars | N/A | Fill name with underscores, not spaces. |
| Latitude | f10.5 | -99.99999 | decimal degrees. South is negative. |
| Longitude | f11.5 | -999.99999 | decimal degrees. West is negative. |
| Elevation | f7.2 | -999.99 | meters |
| Sensor Height | f7.2 | -999.99 | meters; Height of sensor. Positive above ground level. Negative below ground. |
| Station Pressure | f7.2 | -999.99 | hPa (mb). This is the pressure at the reported sensor height |
| Station Pressure Flag | 1 char | M | See Flag values. |
| Air Temperature | f7.2 | -999.99 | Celsius |
| Air Temperature Flag | 1 char | M | See Flag values. |
| Dew Point Temperature | f7.2 | -999.99 | Celsius. Equations. |
| Dew Point Temperature Flag | 1 char | M | See Flag values. |
| Relative Humidity | f7.2 | -999.99 | Percent. Equations. |
| Relative Humidity Flag | 1 char | M | See Flag values. |
| Specific Humidity | f7.2 | -999.99 | g/kg; Note g/kg = (kg/kg)*1000.0. Equations. |
| Specific Humidity Flag | 1 char | M | See Flag values. |
| Wind Speed | f7.2 | -999.99 | m/s. Equations. |
| Wind Speed Flag | 1 char | M | See Flag values. |
| Wind Direction | f7.2 | -999.99 | degrees. Equations. |
| Wind Direction Flag | 1 char | M | See Flag values. |
| U Wind Component | f7.2 | -999.99 | m/s. Equations. |
| U Wind Component Flag | 1 char | M | See Flag values. |
| V Wind Component | f7.2 | -999.99 | m/s. Equations. |
| V Wind Component Flag | 1 char | M | See Flag values. |

## 4.2 Mapping CEOP reference site metadata and ISO 19115 metadata

According to CEOP reference site data set procedures report [14] and ISO 19115 core metadata, we define mapping between the CEOP metadata and ISO metadata as shown in Table 9. By this mapping, some items can be reflected in ISO core metadata, and some are reflected in ISO non-core metadata. To fill the gap between the existing CEOP metadata and the ISO metadata, we add some metadata in CEOP. Table 10 shows the results of additional items required for the CEOP reference site metadata. These additional items will be helpful for use to decide the availability of data sets.

Table 9. Mapping CEOP reference site metadata and ISO 19115 metadata

| CEOP content | Item | ISO package | Class | Attribute | Description |
| --- | --- | --- | --- | --- | --- |
| Contact(s) | Name(s) of reference site Contact(s) | Citation and responsible party information | CI_ResponsibleParty | organisationName | Name of the responsible organization |
|  | Complete mailing address, telephone/facsimile Nos. | Citation and responsible party information | CI_Contact | address | Physical and email address at which the organization or individual may be contacted. |
|  |  |  |  | phone | Telephone numbers at which the organization or individual may be contacted. |
|  | E-mail address and WWW address (if applicable) | Citation and responsible party information | CI_Contact | onlineResource | On-line information that can be used to contact the individual or organization. |
|  | Similar contact information for data questions (if different than above) | Citation and responsible party information | CI_Contact | contactInstructions | Supplemental instructions on how or when to contact the individual or organization |
| Data set overview | Introduction or abstract | Identification Information | MD_Identification | abstract | Brief narrative summary of the content of the resource(s) |
|  | Time period covered by the data | Identification information | MD_Usage | useageDateTime | Date and time of the first use or range of uses of the resource and/or resource series |
|  | Physical location (including lat/lon/elev) of the measurement or platform, landscape, and soil characteristics | Identification | MD_DataIdentification | geographicBox | Minimum bounding rectangle within which data is available |
|  |  |  |  | geographicDescription | Description of the geographic area within which data is available |
|  |  |  |  | environmentDescription | Description of the dataset in the producer’s processing environment, including items such as the software, the computer operating system, file name, and the dataset size |
|  | Data source if applicable (e.g. for operational data include agency) | Lineage information | LI_Lineage | source | Identification information |
|  |  |  | LI_Source | description | Information about the source data used in creating the data specified by the scope |
|  |  |  |  | sourceCitation | Recommended reference to be used for the source data |
|  |  |  |  | sourceExtent | Information about the spatial, vertical and temporal extent of the source data |
|  | Any World Wide Web address references (i.e. additional documentation such as project WWW site) | Metadata extent information | MD_MetadataExtensionInformation | extensionOnLineResource | Information about on-line sources containing the community profile name and the extended metadata elements. Information for all new metadata elements |
| Instrumentation description | Description of instrumentation (e.g. make, model, height, and relationship to the canopy.) | Lineage information | LI_ProcessStep | description | CharacterString |
|  | Figures (or links), if applicable | Identification information | MD_Identification | graphicOverview | Provides a graphic that illustrates the resource(s) (should include a legend for the graphic) |
|  | Instrumentation specifications (e.g. accuracy, precision, frequency, resolution, etc.) | Data quality information | DQ_Element | +NameofMeaure +measureDescription +results (DQ_QuantitativeResult) |  |
| Data collection and processing | Description of data collection | Lineage information | LI_Lineage | statement | General explanation of the data producer’s knowledge about the lineage of a dataset |
|  | Description of derived parameters and processing techniques used (e.g., sampling interval, averaging methods, etc.) | Lineage information | LI_ProcessStep | description | CharacterString Original Def.: description of the event, including related parameters or tolerances |
|  |  |  |  | rationale | CharacterString Original Def: requirement or purpose for the process step |
| Quality control procedures | Description of quality control procedures including an exact description of data flagging methodology (e.g., range limits, physically based checks, etc.) | Data quality information | LI_ProcessStep | description | CharacterString Original Def.: description of the event, including related parameters or tolerances |
| Gap filling procedures | Description of gap filling procedures | Lineage information | LI_ProcessStep | description | CharacterString Original Def.: description of the event, including related parameters or tolerance |
| Data remarks | PI's assessment of the data (i.e. disclaimers, instrument problems, quality issues, etc.) | Data quality element information | DQ_Element | evaluationMethodDescription | Free text Original def.: description of the evaluation method |
|  |  |  |  | evaluationProcedure | CI_Citation Def: reference to the procedure information |
|  |  |  |  | result | DQ_Result Def: value (or set of values) obtained from applying a data quality measure or the out come of evaluating the obtained value (or set of values) against a specified acceptable conformance quality level |
|  | Missing data periods | Data quality element information | DQ_CompletenessOmission |  | Data absent from the dataset, as described by the scope |
|  | Data intercomparisons, if applicable | Data quality element | LI_ProcessStep | description | CharacterString Original Def.: description of the event, including related parameters or tolerance |
| Reference requirements | Specify how you require your data to be referenced | Constraint information | MD_Constraints | useLimitation | CharacterString Original Def.: limitation affecting the fitness for use of theresource or metadata. Example, “not to be used for navigation” |
| References | List of documents cited in this data set description | Distribution information | MD_Medium | name | Name of the medium on which the resource can be received |

Table 10. Additional items for CEOP reference site metadata

| ISO package | Class | Attribute | Description |
| --- | --- | --- | --- |
| Metadata entity set information | MD_Metadata | fileIdentifier | Unique identifier for this metadata file |
|  | MD_Metadata | language | - Language used for documenting metadata |
|  | MD_Metadata | - characterSet | - Full name of the character coding standard used for the metadata set |
|  | MD_Metadata | contact | Party responsible for the metadata information |
|  | MD_Metadata | dateStamp | Date that the metadata was created |
|  | MD_Metadata | metadataStandardName | Name of the metadata standard (including profile name) used |
|  | MD_Metadata | metadataStandardVersion | Version (profile) of the metadata standard used |
|  | MD_Metadata | referenceSystemInfo | Description of the spatial and temporal reference systems used in the dataset |
|  | MD_Metadata | dataQualityInfo | Provides overall assessment of quality of a resource(s) |
| Identification information Identification information | MD_Identification | citation | Basic information required to uniquely identify a resource or resources |
|  | MD_Identification | pointOfContact | Identification of, and means of communication with, person(s) and organizations(s) associated with the resource(s) |
|  | MD_Identification | resourceFormat | Provides a description of the format of the resource(s) |
|  | MD_DataIdentification | spatialRepresentationType | Method used to spatially represent geographic information |
|  | MD_DataIdentification | spatialResolution | Factor which provides a general understanding of the density of spatial data in the dataset |
|  | MD_DataIdentification | language | Language(s) used within the dataset |
|  | MD_DataIdentification | topicCategory | Main theme(s) of the dataset |
|  | MD_DataIdentification | extent | Additional extent information including the bounding polygon, vertical, and temporal extent of the dataset |
|  | MD_ServiceIdentification | typeProperties | Attributes that can be queried for this service type. Standardized in a service type registry |
| Data quality information | DQ_DataQuality | lineage | Non-quantitative quality information about the lineage of the data specified by the scope |
| Distribution information | MD_Distribution | distributionFormat | Provides a description of the format of the data to be distributed |
|  | MD_Format | name | Name of the data transfer format(s) |
|  | MD_Format | version | Version of the format (date, number, etc.) |
| Citation and responsible party information | CI_Citation | title | Name by which the cited resource is known |
|  | CI_Citation | date | Reference date for the cited resource |
|  | CI_Citation | citedResponsibleParty | Name and position information for an individual or organization that is responsible for the resource |
|  | - CI_ResponsibleParty | - contactInfo | - Address of the responsible party |
|  | CI_OnLineResource | linkage | Location (address) for on-line access using a Uniform Resource Locator address or similar addressing scheme such as http://www.statkart.no/isotc211 |

# 5. GRIB metadata

GRidded In Binary (GRIB) is the standard format for the storage and interchange of meteorological data. A new standard for the transmission of gridded data has been developed by the World Meteorological Organization (WMO) and was declared operationally in November 2001.

## 5.1 WMO core metadata

Table 11 provides an overview of the WMO community core metadata profile suitable for use by decision makers and users.

This profile provides a general definition for directory searches and exchanges that should be applicable to a wide variety of WMO datasets. It does not specify how these metadata should be archived or presented to users. It also does not specify any particular implementation and could be implemented as a database, a flat file, or any other suitable mechanism. However, XML is recommended as the standard for exchange. The comprehensive and technical details required for implementation of this standard in XML are provided in the WMO core metadata XML schema. In the XML schema, class names (such as MD_Metadata) are reserved for data types. These data type names do not appear in the XML instance. Instead, the classes are referenced by the name of an object (e.g. metadata) which is an instance of the class (MD_Metadata).

The list below defines a minimum set of information to describe data for WMO exchange and is not exhaustive [15]. Of the core elements listed, those in bold are required, with all others being optional.

Table 11. WMO core metadata

| Generic name | ISO field / class name and reference lines | Definition |
| --- | --- | --- |
|  | MD_Metadata (1) | Root entity which defines metadata about a resource or resources |
| Metadata ID | fileIdentifier (2) | Unique identifier for this metadata file |
| Metadata language | language (3) | Language used for documenting metadata |
| Metadata char. set | characterSet (4) | Character set of this metadata item (Default of ISO 10646-1) |
| Metadata contact | contact (8) | Party responsible for the metadata information |
|  | CI_ResponsibleParty (see 376 below) | Identification of, and means of communication with, person(s) and organizations associated with the dataset |
| Metadata date | dateStamp (9) | Date that the metadata was created |
| Metadata name | metadataStandardName (10) | Name of the metadata standard (including profile name) used |
| Metadata Version | metadataStandardVersion (11) | Version (profile) of the metadata standard used |
|  | referenceSystemInfo (13) | Description of the spatial and temporal reference systems used in the dataset |
| Data Reference System | MD_ReferenceSystem (188) | Information about the reference systems used (temporal, coordinate and geographic) |
|  | rsID (189) | Name of reference system |
|  | authority (208) | Person or party responsible for maintenance of the namespace |
|  | CI_ResponsibleParty (376) (see below) | Identification of, and means of communication with, person(s) and organizations associated with the dataset |
|  | ReferenceDescription (207) | Description of the Reference System |
|  | IdentificationInfo (15) | Basic information about the resource(s) to which the metadata applies |
| Data information | MD_DataIdentification (see 36 below) | Information required to identify a dataset |
|  | DistributionInfo (17) | Provides information about the distributor of and options for obtaining the resource(s) |
| Data distribution Information | MD_Distribution (see 272 below) | Information about the distributor of and options for obtaining the resource |
|  | DataQualityInfo (18) | Provides overall assessment of quality of a resource(s) |
| Data Lineage or Quality | LI_Lineage (see 84 below) | Information about the data lineage or quality |
|  | MD_DataIdentification (36) | Information required to identify a dataset |
|  | Citation (24) | Citation data for the resource(s) |
| Title | title (362) | Name by which the cited resource is known |
| Reference Date | date (362) | Reference date for the cited resource |
|  | CI_Date (see 395 below) | Reference date and event used to describe it |
| Identifier | Identifier (367) | Unique identifier for the resource |
|  | IdentifierType (368) | Reference form of the unique identifier (ID) |
| Abstract | abstract (25) | Brief narrative summary of the content of the resource(s) |
| Dataset Contact | pointOfContact (29) | Identification of, and means of communication with, person(s) and organizations(s) associated with the resource(s) |
|  | CI_ResponsibleParty (see 376 below) | Identification of, and means of communication with, person(s) and organizations associated with the dataset |
|  | resourceMaintenance (30) | Provides information about the frequency of resource updates, and the scope of those updates |
| Update frequency | MaintenanceAndUpdateFrequency (145) | Frequency with which changes and additions are made to the resource after the initial resource is completed |
|  | maintenanceNote (150) | Information regarding specific requirements for maintaining the resource |
| Access Rights or Restrictions | resourceConstraints (35) | Provides information about constraints which apply to the resource(s) |
|  | MD_Constraints (69) (see below) | Restrictions on the access and use of a resource or metadata |
| Spatial Resolution | spatialResolution (38) | Factor which provides a general understanding of the density of spatial data in the dataset |
|  | spatialRepresentationType (37) | Method used to spatially represent geographic information |
| Language | language (39) | Language(s) used within the dataset |
| Character set | characterSet (40) | Full name of the character coding standard used for the dataset |
| Topic Category | topicCategory (41) | Main theme(s) of the datset |
| Keywords | descriptiveKeywords (33) | Provides category keywords, their type, and reference source |
| Date or period | CI_Date (395) | Reference date and event used to describe it |
|  | date (396) | Reference date for the cited resource |
|  | DateType (397) | Event used for reference date |
| Responsible Party | CI_ResponsibleParty (376) | Identification of, and means of communication with, person(s) and organizations associated with the dataset |
| Organisation | organisationName (378) | Name of the responsible organization |
| Org. role | role(381) | Function performed by the responsible party |
| Individual name | individualName (377) | Name of the responsible person- surname, given name, title separated by a delimiter |
| Position | positionName (379) | Role or position of the responsible person |
|  | contactInfo (380) | Address of the responsible party |
|  | CI_Contact (389) | Information required to enable contact with the responsible person and/or organization |
| Phone number | phone (390) | Telephone numbers at which the organization or individual may be contacted |
|  | voice (410) | Telephone number by which individuals can speak to the responsible organization or individual |
| Fax number | Facsimile (411) | Telephone number of a facsimile machine for the responsible organization or individual |
| Address | address (391) | Physical and email address at which the organization or individual may be contacted |
|  | deliveryPoint (383) | Address line for the location (as described in ISO 11180, Annex A) |
|  | city (384) | City of the location |
|  | postalCode (386) | ZIP or other postal code |
|  | Country (387) | Country of the physical address |
| E-mail address | electronicMailAddress (388) | Address of the electronic mailbox of the responsible organization or individual |
| Web Site | onLineResource (392) | On-line information that can be used to contact the individual or organization |
|  | EX_Extent (336) | Information about spatial, vertical, and temporal extent |
|  | Description (337) | Spatial and temporal extent for the referring object |
| Vertical Extent | Ex_VerticalExtent (354) | Extent with respect to date/time and spatial boundaries |
|  | minimumValue (357) | Lowest vertical extent contained in the dataset |
|  | maximumValue (358) | Highest vertical extent contained in the dataset |
|  | unitOfMeasure (359) | Vertical units used for vertical extent information |
|  | verticalDatum (360) | Provides information about the origin from which the maximum and minimum elevation values are measured |
| Geographic Extent | EX_GeographicExtent (341) | Geographic area of the dataset |
| Geographic name | EX_GeographicDescription (350) | Description of the geographic area using identifiers |
|  | geographicIdentifier (351) | Identifier used to represent a geographic area |
| Bounding box | EX_GeographicBoundingBox (345) | Geographic position of the dataset |
|  | westBoundLongitude (346) | Western-most coordinate of the limit of the dataset extent, expressed in longitude in decimal degrees (positive east) |
|  | eastBoundLongitude (347) | Eastern-most coordinate of the limit of the dataset extent, expressed in longitude in decimal degrees (positive east) |
|  | southBoundLatitude (348) | Southern-most coordinate of the limit of the dataset extent, expressed in latitude in decimal degrees (positive north) |
|  | northBoundLatitude (349) | Northern-most, coordinate of the limit of the dataset extent expressed in latitude in decimal degrees (positive north) |
| Bounding polygon | EX_BoundingPolygon (343) | Boundary enclosing the dataset, expressed as the closed set of (x,y) coordinates of the polygon (last point replicates first point) |
|  | Polygon (342) | Sets of points defining the bounding polygon |
| Temporal Extent | EX_TemporalExtent (352) | Time period covered by the content of the dataset |
|  | Extent (353) | Date and time for the content of the dataset |
|  | beginDateTime (new) | Beginning date of the data in the dataset |
|  | endDateTime (new) | Ending date of data in the dataset |
|  | dataFrequency (new) | Observing frequency of the data in the dataset (code: WMO_DataFrequencyCode) |
| Access Rights or Restrictions | MD_Constraints (69) | Restrictions on the access and use of a resource or metadata |
|  | useLimitation (70) | Limitation affecting the fitness for use of the resource |
|  | MD_LegalConstraints (71) | Restrictions and legal prerequisites for accessing and using the resource |
|  | accessConstraints (72) | Access constraints applied to assure the protection of privacy or intellectual property, and any special restrictions or limitations on obtaining the resource |
|  | useConstraints (73) | Constraints applied to assure the protection of privacy or intellectual property, and any special restrictions or limitations or warnings on using the resource |
|  | otherConstraints (74) | Other restrictions and legal prerequisites for accessing and using the resource |
|  | MD_SecurityConstraints (75) | Handling restrictions imposed on the resource for national security or similar security concerns |
|  | classification (76) | Name of the handling restrictions on the resource |
|  | userNote (77) | Explanation of the application of the legal constraints or other restrictions and legal prerequisites for obtaining and using the resource |
|  | classificationSystem (78) | Name of the classification system |
|  | handlingDescription (79) | Additional information about the restrictions on handling the resource |
|  | MD_Distribution (272) | Information about the distributor of and options for obtaining the resource |
|  | distributionFormat (273) | Provides a description of the format of the data to be distributed |
| Format name | name (287) | Name of the data transfer format(s) |
| Format version | version (288) | Version of the format (date, number, etc.) |
|  | TransferOptions (275) | Provides information about technical means and media by which a resource is obtained from the distributor |
| On-line source | OnLine (279) | Information about online sources from which the resource can be obtained |
|  | linkage (399) | Location (address) for on-line access using a Uniform Resource Locator address or similar addressing scheme such as http://www.statkart.no/isotc211 |
|  | WMO_Source (new) | WMO centre identifier |
| Off-line media | Offline (280) | Information about offline media on which the resource can be obtained |
|  | name (294) | Name of the medium on which the resource can be received |
| Processing Level | LI_Lineage (84) | Information about the events or source data used in constructing the data specified by the scope or lack of knowledge about lineage |
|  | statement (85) | General explanation of the data producer’s knowledge about the lineage of a datase |
|  | processStep (86) | Information about an event in the creation process for the data specified by the scope |
|  | source (87) | Information about the source data used in creating the data specified by the scope |
| Reference System | MD_ReferenceSystem (188) | Information about the reference system |
|  | rsID (189) | Name of reference system |
|  | authority (208) | Person or party responsible for maintenance of the namespace |
|  | CI_ResponsibleParty (376) (see above) | Identification of, and means of communication with, person(s) and organizations associated with the dataset |
|  | code (209) | Alphanumeric value identifying an instance in the namespace |

## 5.2 Example of GRIB metadata

<?xml version="1.0" encoding="UTF-8"?>

<!-- XML file generated by XML Spy v4.1 U (http://www.xmlspy.com)

Gil Ross Met Office UK-->

<metaData xmlns="http://www.wmo.ch/www/metadata" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="http://www.wmo.ch/www/metadata

http://www.wmo.ch/www/metadata/WMO19115_metadata_v0_1.xsd">

<metadataLanguage>en</metadataLanguage>

<metadataCharacterSet>utf8</metadataCharacterSet>

<metadataContact>

<organisationName>Met Office</organisationName>

<role>resourceProvider</role>

<positionName>Customer Call Centre</positionName>

<contactInfo>

<phone>

<voice>0845 300 0300</voice>

<voice>+44 (0)1344 855680 </voice>

<facsimile>0845 300 1300</facsimile>

<facsimile>+44 (0)1344 855681</facsimile>

</phone>

<address>

<deliveryPoint> London Road</deliveryPoint>

<city>Bracknell</city>

<administrativeArea>Berkshire</administrativeArea>

<postalCode>RG12 2SZ</postalCode>

<country>United Kingdom</country>

<electronicMailAddress>enquiries@metoffice.com

</electronicMailAddress>

</address>

<onlineAddress> http://www.metoffice.com</onlineAddress>

</contactInfo>

</metadataContact>

<metadataDateStamp>2002-05-21</metadataDateStamp>

<metadataStandardName>WMO19115_metadata_v0_1</metadataStandardName>

<metadataStandardVersion>0.1</metadataStandardVersion>

<referenceSystemInfo>

<referenceSystemIdentifier>Plat Caree</referenceSystemIdentifier>

<referenceDescription>Global regularly spaced latitude longitude grid</referenceDescription>

</referenceSystemInfo>

<identificationInfo>

<citation>

<resourceTitle>Complete Met Office Global Model forecast composed of GRIB bulletins</resourceTitle>

<resourceIdentifier>TBA</resourceIdentifier>

</citation>

<abstract>Full set of GRIB bulletins prepared for a single Met Office Global model forecast</abstract>

<pointOfContact>

<organisationName>Met Office</organisationName>

<role>resourceProvider</role>

<positionName>Customer Call Centre</positionName>

<contactInfo>

<phone>

<voice>0845 300 0300 </voice>

<voice>+44 (0)1344 855680</voice>

<facsimile>0845 300 1300</facsimile>

<facsimile>+44 (0)1344 855681</facsimile>

</phone>

<address>

<deliveryPoint> London Road</deliveryPoint>

<city>Bracknell</city>

<administrativeArea>Berkshire</administrativeArea>

<postalCode>RG12 2SZ</postalCode>

<country>United Kingdom</country>

<electronicMailAddress>enquiries@metoffice.com</electronicMailAddress>

</address>

</contactInfo>

</pointOfContact>

<resourceMaintenance>

<maintenanceAndUpdateFrequency>12-hourly</maintenanceAndUpdateFrequency>

<maintenanceNote>disseminated at 4 hours past the reference date/time</maintenanceNote>

</resourceMaintenance>

<resourceConstraints>

<useLimitation>This is provided under aegis of WMO, or alternatively under written contract</useLimitation>

<legalConstraints>

<useConstraints>intellectualPropertyRights </useConstraints>

</legalConstraints>

</resourceConstraints>

<spatialRepresentationType>grid</spatialRepresentationType>

<topicCategory>climatologyMeteorologyAtmosphere </topicCategory>

<descriptiveKeywords>NWP Model Analysis Forecast GRIB Bulletin Pressure Height Wind Maximum Speed Direction Temperature Relative Humidity Precipitation Amount Rate Cloud Liquid Ice Tropopause </descriptiveKeywords>

<referenceDate>

<date>2002-05-01T00:00:00</date>

<dateType>referenceDate</dateType>

</referenceDate>

<dataExtent>

<description>Global Grid 2.5 degree latitude and 2.5 degree longitude steps, in 6 sectors, one sector per GRIB </description>

<geographicElement>

<geographicBoundingBox>

<westBoundLongitude>-180 </westBoundLongitude>

<eastBoundLongitude>180</eastBoundLongitude>

<southBoundLatitude>-90</southBoundLatitude>

<northBoundLatitude>90</northBoundLatitude>

</geographicBoundingBox>

</geographicElement>

</dataExtent>

<dataExtent>

<description>Validity Time Range</description>

<temporalElement>

<beginDateTime>T+00</beginDateTime>

<endDateTime>T+120</endDateTime>

<dataFrequency>6-hourly</dataFrequency>

</temporalElement>

</dataExtent>

<dataExtent>

<description>Vertical Range also includes surface measures such as rainfall accumulations</description>

<verticalElement>

<minimumValue>100</minimumValue>

<maximumValue>1000</maximumValue>

<unitOfMeasure>hectoPascals</unitOfMeasure>

</verticalElement>

</dataExtent>

</identificationInfo>

<distributionInfo>

<distributionFormat>

<formatName>WMO GRIB FM92</formatName>

<formatVersion>version 1 GRIB</formatVersion>

</distributionFormat>

<transferOptions>

<transferSize>21</transferSize>

<onLineSource>http://www.metoffice.com</onLineSource>

<WMO_Source>WMO GTS on reference date </WMO_Source>

<offlineMedium>digitalLinearTape</offlineMedium>

</transferOptions>

</distributionInfo>

<dataQualityInfo>

<lineageStatement>This is the unmodifed output from the Met Office Global Model on the declared date</lineageStatement>

<dataProcessInfo>The NWP Output has been interpolated to the GRIB grid and compressed using GRIB methods</dataProcessInfo>

<dataSourceInfo>The output is from an internally consistent Numerical Weather Predicton model initiated from the analysis at the reference date</dataSourceInfo>

</dataQualityInfo>

</metaData>

# 6. MOLTS metadata

## 6.1 GMAO MOLTS metadata

Global Modeling and Assimilation Office (GMAO) is now providing Model Output Data for supporting CEOP [17] (http://gmao.gsfc.nasa.gov/sci_research/CEOP/README1st.php). These data are from the GEOS3 operational data assimilation system for the period 1 July, 2001 to 30 September, 2001 corresponding to the CEOP Enhanced Observing Period-1 (EOP-1).

The GMAO will provide operational analyses for CEOP. The data for the first CEOP EOP-1 are available as are the Model Output Time Series (MOLTS) for the CEOP reference sites as follows.

• Temporal Coverage

Start Date: 2001-07-01
Stop Date: 2004-09-30

• Geographic Coverage

Southernmost Latitude: 90.0S
Westernmost Longitude: 180.0W
Northernmost Latitude: 90.0N
Easternmost Longitude: 180.0E

• Location Keywords

GLOBAL

• Data Resolution

Latitude Resolution: 1 degree
Longitude Resolution: 1 degree

• Parameters

HYDROSPHERE > SNOW/ICE > SNOW DEPTH

CRYOSPHERE > SEA ICE > SNOW DEPTH

BIOSPHERE > VEGETATION > VEGETATION INDEX > GREENNESS INDEX

BIOSPHERE > VEGETATION > LEAF CHARACTERISTICS > LEAF AREA INDEX

BIOSPHERE > VEGETATION > PHOTOSYNTHETICALLY ACTIVE RADIATION > PAR

ATMOSPHERE > ATMOSPHERIC RADIATION > OUTGOING LONGWAVE RADIATION

ATMOSPHERE > ATMOSPHERIC RADIATION > LONGWAVE RADIATION

ATMOSPHERE > PRECIPITATION > SNOW > SNOWFALL

ATMOSPHERE > PRECIPITATION > RAIN

ATMOSPHERE > PRECIPITATION > PRECIPITATION AMOUNT

ATMOSPHERE > ATMOSPHERIC PRESSURE > SURFACE PRESSURE

ATMOSPHERE > ATMOSPHERIC PRESSURE > PRESSURE TENDENCY

LAND SURFACE > TOPOGRAPHY > SURFACE ROUGHNESS

ATMOSPHERE > ATMOSPHERIC WINDS > WIND STRESS

ATMOSPHERE > ATMOSPHERIC WATER VAPOR > PRECIPITABLE WATER

ATMOSPHERE > ATMOSPHERIC WINDS > SURFACE WINDS

ATMOSPHERE > ATMOSPHERIC RADIATION > ATMOSPHERIC HEATING

ATMOSPHERE > ATMOSPHERIC RADIATION > ALBEDO

ATMOSPHERE > ATMOSPHERIC WATER VAPOR > WATER VAPOR

ATMOSPHERE > ATMOSPHERIC WATER VAPOR > HUMIDITY > SPECIFIC HUMIDITY

LAND SURFACE > LAND TEMPERATURE > LAND SURFACE TEMPERATURE

ATMOSPHERE > ATMOSPHERIC TEMPERATURE > SURFACE AIR TEMPERATURE

ATMOSPHERE > ATMOSPHERIC TEMPERATURE > AIR TEMPERATURE

ATMOSPHERE > ATMOSPHERIC WATER VAPOR > EVAPORATION

ATMOSPHERE > CLOUDS > CLOUD TOP TEMPERATURE

ATMOSPHERE > CLOUDS > CLOUD TOP PRESSURE

ATMOSPHERE > CLOUDS > CLOUD OPTICAL DEPTH/THICKNESS

ATMOSPHERE > CLOUDS > CLOUD AMOUNT/FREQUENCY

ATMOSPHERE > ATMOSPHERIC RADIATION > SHORTWAVE RADIATION

ATMOSPHERE > ATMOSPHERIC RADIATION > INCOMING SOLAR RADIATION

• ISO Topic Category

CLIMATOLOGY/METEOROLOGY/ATMOSPHERE

BIOTA

OCEANS

INLAND WATERS

GEOSCIENTIFIC INFORMATION

ELEVATION

• Platform

GCM > General Circulation Model

MODELS

• Project

CEOP > Coordinated Enhanced Observing Period

GEWEX > Global Energy and Water Cycle Experiment

• Entry ID

CEOP_DAO

• Keywords

CEOP > Coordinated Enhanced Observing Period

GEWEX > Global Energy and Water Cycle Experiment

• Dataset Progress

IN WORK

• Originating Center

NASA/DAO

• Data Center

Data Center Name: NASA/GSFC/GMAO > Global Modeling and Assimilation, Goddard Space Flight Center, NASA 
Data Center URL: http://gmao.gsfc.nasa.gov/

• Personnel

Name: MICHELE, DR. RIENECKER
Email: mrienecker@gmao.gsfc.nasa.gov
Contact Address:

Head, Global Modeling and Assimilation Office

NASA Goddard Space Flight Center, Code 900.3 
City: Greenbelt
Province or State: MD
Postal Code: 20771

• Distribution

Distribution Media: ftp
Fees: none

• Related URL

Content Type: RELATED DATA SET DESCRIPTION
URL: http://gmao.gsfc.nasa.gov/sci_research/CEOP/index.php 
Description: Access to additional documentation

• Reference

Chou, M. D., 1984: Broadband water vapor transmission functions for atmospheric IR flux computations. J. Atmos. Sci., 41, 1775-1778. etc.

• Metadata Name and Version

Metadata Name: CEOS IDN DIF
Metadata Version: VERSION 9.0

• Creation and Review Dates

DIF Creation Date: 2003-05-27

## 6.2 MOLTS metadata

On the basis of above GMAO MOLTS metadata, some items are required to append in the design of MOLTS metadata, including

(1) Identifiers

Location ID

(2) Time

Valid Date/Time

Time resolution

(3) Location of grid point

Point location of model simulation

(4) MOLTS output variables [18]

Atmosphere Variables

Vertically Integrated Atmos. Variables

Surface Variables

Subsurface Variables

Bottom of Subsurface Variables

Miscellaneous

Table 12 shows the design results of MOLTS variables to ISO mapping.

Table 12. MOLTS variables to ISO mapping

| GCMD record | Record content/example | ISO Class | Attribute | Description |
| --- | --- | --- | --- | --- |
| Data set citation | Dataset Creator | MD_Identification | citation | Citation data for the resource(s) |
|  | Dataset Title | MD_Identification | abstract | brief narrative summary of the content of the resource(s) |
|  | Dataset Release Date |  |  |  |
|  | Dataset Release Place |  |  |  |
|  | Dataset Publisher | MD_Identification | pointOfContact | identification of, and means of communication with, person(s) and organization(s) associatedwith the resource(s) |
|  | Online Resource | MD_Identification | resourceFormat | provides a description of the format of the resource(s) |
| Temporal coverage | Start Date Stop Date | EX_TemporalExtent | extent | Date and time for the content of the dataset |
| Geographic coverage | Southernmost Latitude | EX_GeographicBoundingBox | southBoundLatitude | Southern-most coordinate of the limit of the dataset extent, expressed in latitude in decimal degrees (positive north) |
|  | Westernmost Longitude | EX_GeographicBoundingBox | westBoundLongitude | Western-most coordinate of the limit of the dataset extent, expressed in longitude in decimal degrees (positive east) |
|  | Northernmost Latitude | EX_GeographicBoundingBox | northBoundLatitude | Northern-most, coordinate of the limit of the dataset extent expressed in latitude in decimal degrees (positive north) |
|  | Easternmost Longitude | EX_GeographicBoundingBox | eastBoundLongitude | Eastern-most coordinate of the limit of the dataset extent, expressed in longitude in decimal degrees (positive east) |
| Location keywords | GLOBAL | EX_GeographicDescription | geographicIdentifier | Identifier used to represent a geographic area |
| Data resolution | Latitude Resolution Longitude Resolution | MD_DataIdentification | spatialResolution | Factor which provides a general understanding of the density of spatial data in the dataset |
| Parameters | HYDROSPHERE > SNOW/ICE > SNOW DEPTH CRYOSPHERE > SEA ICE > SNOW DEPTH | MD_DataIdentification | topicCategory | Main theme(s) of the dataset |
| ISO topic category | CLIMATOLOGY / METEOROLOGY / ATMOSPHERE BIOTA OCEANS INLAND WATERS GEOSCIENTIFIC INFORMATION ELEVATION | MD_TopicCategoryCode <<Enumeration>> | ClimatologyMeteorology Atmosphere, inlandWaters, oceans etc. | High-level geographic data themathic classification to assist in the grouping and search of available geographic data sets. Can be used to group keywords as well. Listed examples are not exhaustive. |
| Platform | GCM > General Circulation Model MODELS | MD_CoverageDescription | attributeDescription | Description of the attribute described by the measurement value |
| Project | CEOP > Coordinated Enhanced Observing Period GEWEX > Global Energy and Water Cycle Experiment | LI_Lineage | source | Information about the source data used in creating the data specified by the scope |
| Entry ID | CEOP_DAO | MD_Identifier | authority | Person or party responsible for maintenance of the namespace |
| Keywords | MOLTS Model Output Location Time Series Goddard Earth Observing System GEOS-3 CEOP model data output TEMS GTOS G3OS | MD_Keywords | keyword | Commonly used word(s) or formalised word(s) or phrase(s) used to describe the subject |
| Dataset progress | IN WORK | MD_DataIdentification | supplementalInformation | Any other descriptive information about the dataset |
| Originating center | NASA/DAO | MD_Distributor | distributorContact | party from whom the resource may be obtained. This list need not be exhaustive |
| Data center | Data Center Name | MD_Distribution | distributor | provides information about the distributor |
|  | Data Center URL | MD_DigitalTransferOptions | onLine | information about online sources from which the resource can be obtained |
| Distribution | Distribution Media | MD_Medium | name | Name of the medium on which the resource can be received |
|  | Fees | MD_StandardOrderProcess | fees | Fees and terms for retrieving the resource. Include monetary units |
| Personnel | Name | CI_ResponsibleParty | individualName | Name of the responsible person- surname, given name, title separated by a delimiter |
|  | Role | CI_ResponsibleParty | role | Function performed by the responsible party |
|  | Email | CI_Address | electronicMailAddress | Address of the electronic mailbox of the responsible organization or individual |
|  | Contact Address | CI_Address | address | Physical and email address at which the organization or individual may be contacted |
|  | City | CI_Address | city | City of the location |
|  | Province or State | CI_Address | administrativeArea | State, province of the location |
|  | Postal Code | CI_Address | postalCode | ZIP or other postal code |
|  | Country | CI_Address | country | Country of the physical address |
| Related URL | Content Type | CI_OnlineResource | name | Name of the online resource |
|  | URL | CI_OnlineResource | linkage | Location (address) for on-line access using a Uniform Resource Locator address or similar addressing scheme such as http://www.statkart.no/isotc211 |
|  | Description: Access to additional documentation | CI_OnlineResource | description | Detailed text description of what the online resource is/does |
| Reference | Chou, M. D., 1984: Broadband water vapor transmission functions for atmospheric IR flux computations. J. Atmos. Sci., 41, 1775-1778 | MD_FeatureCatalogueDescription | featureCatalogueCitation | Complete bibliographic reference to one or more external feature catalogues |
| Metadata name and version | Metadata Name | MD_Metadata | metadataStandardVersion | Name of the metadata standard (including profile name) used |
|  | Metadata Version | MD_Metadata | metadataStandardVersion | Version (profile) of the metadata standard used |
| Creation and review dates | DIF Creation Date | CI_Citation | editionDate | Date of the edition |
| Identifiers (new) | Location ID | EX_GeographicDescription | geographicIdentifier | Identifier used to represent a geographic area |
| Time (new) | Valid Date/Time | EX_TemporalExtent | extent | Date and time for the content of the dataset |
|  | Time resolution |  |  | Note: need to add |
| Location of grid point (new) | Point location of model simulation | Ex_BoundingPolygon | Polygon[1..x]: GM_Object | Center point of a grid originally used for model simulation |
| MOLTS variables (new) | Atmosphere Variables, Vertically Integrated Atmos. Variables, Surface Variables, Subsurface Variables, Bottom of Subsurface Variables, Miscellaneous | MD_CoverageDescription | attributeDescription | Description of the attribute described by the measurement value |
|  |  | MD_CoverageDescription | contentType | Type of information represented by the cell value |

# References

[1] ISO/TC 211 Secretariat. ISO 19115: Geographic information – Metadata

[2] ISO/TC 211 Secretariat. ISO 19139: Geographic information - Metadata – Implementation specifications

[3] JAXA, AMSR Data Input Toolkit (ADIT) User's Guide Version 1.03, 2003

[4] NASDA, AMSR-E Data Users Handbook 2nd Edition, 2003

[5] NASDA, Appendix. 3-1 AMSR-E Level 1 format description (NEB-00011C)

[6] NASDA, Appendix. 3-2 AMSR-E Level 2 format description (NDX-000272A)

[7] NASDA, Appendix. 3-3 AMSR-E Level 2 Map format description (NDX-000273A)

[8] NASDA, Appendix. 3-4 AMSR-E Level 3 format description (NDX-000274A)

[9] DODS Index of /CEOP/Satellite, http://dodsdev.gso.uri.edu/dods-3.4/nph-dods/CEOP/Satellite/F13SSMI1B_200107152131_wpo_low.dat.html

# [10] CEOP surface meteorological and radiation dataset format definition, http://www.joss.ucar.edu/ghp/ceopdm/refdata_report/ceop_sfc_met_format.html

# [11] CEOP flux dataset format definition, http://www.joss.ucar.edu/ghp/ceopdm/refdata_report/ceop_flux_format.html

# [12] CEOP soil temperature and soil moisture dataset format definition, http://www.joss.ucar.edu/ghp/ceopdm/refdata_report/ceop_soils_format.html

[13] CEOP meteorological tower dataset format definition, http://www.joss.ucar.edu/ghp/ceopdm/refdata_report/ceop_met_tower_format.html

[14] CEOP reference site data set procedures report, http://www.joss.ucar.edu/ghp/ceopdm/refdata_report/

[15] DRAFT WMO core metadata, http://www.wmo.int/web/www/metadata/WMO-core-metadata-toc.html.

[16] A Guide to WMO GRIB Edition 1, http://dss.ucar.edu/docs/formats/grib/gribdoc/

[17] Global Modeling and Assimilation Office (GMAO) Model Output Data, http://gcmd.gsfc.nasa.gov/cgi-bin/md/zgate?present+13480+Default+2+1+F+1.2.840.10003.5.109.3+CEOP_DAO

[18] Appendix A. Model output variables requested by CEOP, http://ceop.net/